| PRE-START - Canopy LOCKED - Park Brake ON - Circuit breakers CHECK - Master Switch ON - Fuel Switch LOWEST TANK - Throttle IDLE - Propeller FULL FINE - Avionics OFF - Fuel pump ON - Fuel Pressure Warn CK OFF - Fuel pump OFF - Choke if cold engine ON - “Clear Prop” ANNOUNCE - Starter ENGAGE AFTER START - Oil Pressure >1.5 BAR - Choke OFF - Throttle 820 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ± Range of plus or minus ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp ≥50°C - Park Brake ON - Choke OFF - Fuel FULLEST TANK - Throttle 1700 RPM - Ign R/L ≤120 RPM Δ ≤50 RPM - Propeller CYCLE x3 - Propeller Δ 200±50RPM - Carburettor heat ON→OFF - Idle CHECK ≥580 RPM - Throttle SET 820 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture CHOKE OFF - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Flaps TKOF 17° - Fuel FULLEST TANK&PUMP ON - Instruments CHECK - Switches CIRC BRKERS - Switches AS REQ - Hatches & Harnesses CHECK - Safety/DEP BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture CHOKE OFF - Fuel SUFF QTY & PUMP ON - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Lights OFF - Flaps & Trim SET TKOF SHUT DOWN - Park Brake ON - Throttle MIN RPM - Avionics OFF - Ignition RIGHT ...pause 2 seconds - Ignition OFF - Master Switch OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE&CLEAN |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 78KIAS - Troubleshoot F.A.S.T. - Fuel pump ON - Fuel select CHANGE CHECK - Air CARB HEAT ON - Switches IGNITION BOTH - Switches MASTER ON - Throttle CYCLE - Turn over ENGINE START - Field SELECT - Wind - Obstacles - Size - Shape - Surface - Slope - Sun - Shoots - Surroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - Height RECOVER ≥3000 AGL - Aircraft CONFIG, TRIM - Security HARNESS, OBJECTS - Engine OPS NORMAL, P/T - Location UNPOPUL, FLD AVBL - Lookout TRAFFIC - Listen CHECK FREQ SPEEDS - Vne 165KIAS - Vno 130KIAS - Va 112KIAS - Vfe 90KIAS - Vs0 39KIAS - Vs1 49KIAS - Vx 52KIAS - Vy 65KIAS - Best Glide 78KIAS - Max crosswind 15KT | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - Southport CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE - Model Rotax 912ULS - Max RPM (cont.) 2260RPM - Block Fuel Burn 24L/hr - Min VFR Reserve 45min - Fuel 95RON¦AVGAS100LL - Min Oil Temp 50°C - Max Oil QTY 3.0L - Min Oil QTY 2.55L |
| --- | --- | --- |

|  | VH-BNE - Total Fuel Capacity 120.0L - Total Fuel Capacity 86.4kg - Total Usable Fuel 109.6L - Total Usable Fuel 78.9kg - Basic Empty Weight 521.4kg - Max Take-Off Weight 750kg - Usable Load at full fuel 149.7kg - Empty Arm 434.0mm - Permitted CG min. 427mm - Permitted CG max. 523mm - Min Crew Weight 54.9kg - Max Baggage Weight 40kg - Max LF (no flaps) +4G - Min LF (no flaps) -2G - Max LF (flaps) +2G - Min LF (flaps) 0G - Max Angle of Bank 60° - Propeller MTV-21-A/175-05 ⌀1750mm - Primary Flight Display Garmin G500 - Maximum propeller speed 2260RPM - Flap positions 0° 17° 35° |  |
| --- | --- | --- |