| PRE-START - Park Brake ON - Doors CLOSED - Circuit Breakers CHECK - Master Switch ON - Fuel Selector ON if engine is COLD, prime - Primer STROKE x2 - Fuel Mixture RICH - Throttle ¼″ OPEN - Magnetos BOTH - Avionics OFF - Propeller Area CHECK - “Clear Prop” ANNOUNCE - Starter ENGAGE AFTER START - Oil Pressure RISING/GREEN - Throttle 1000 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp IN GREEN - Park Brake ON - Throttle 1700 RPM - Vacuum Gauge CHECK - Ignition R CHECK ≤125 RPM - Ignition L CHECK ≤125 RPM - Ignition R/L Δ ≤50 RPM - Carb Heat ON→OFF(RPM drop) - Ammeter CHECK CHARGE - Idle CHECK ≥600 RPM - Throttle 1000 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture RICH - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Flaps SET TKOF 10° - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK - T/O Safety/DEP/TEM BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture RICH - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Flaps 0° - Lights OFF SHUT DOWN - Park Brake ON - Trim SET TKOF - Throttle MIN RPM - Avionics OFF - Fuel Mixture IDLE CUTOFF - Magnetos OFF - Master Switch OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 60KIAS - Troubleshoot F.A.S.T. - F uel primer LOCKED - F uel selector BOTH - A ir CARB HEAT ON - S witches ON - T hrottle CYCLE - T urn over ENGINE START - Field SELECT - W ind - O bstacles - S ize - S hape - S urface - S lope - S un - S hoots - S urroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE - Max Glide Ratio 3NM/2000ft - Wind NIL - Flaps RETRACTED - Propeller WINDMILLING | STALL ENTRY - Clearing Turn AS REQ - H eight RECOVER ≥3000 AGL - A ircraft CONFIG, TRIM - S ecurity HARNESS, OBJECTS - E ngine OPS NORMAL, P/T - L ocation UNPOPUL, FLD AVBL - L ookout TRAFFIC - L isten CHECK FREQ SPEEDS & LIMITS - Vne 149KIAS - Vno 111KIAS - Va1670lb 104KIAS - Va1500lb 98KIAS - Va1350lb 93KIAS - Vfe 85KIAS - Vs0 35KIAS - Vs1 40KIAS - Vx 55KIAS - Vy 67KIAS - Best Glide 60KIAS - Max crosswind 12KT - Load Factorcruise +4.4G/-1.76G - Load Factorflaps +3.5G - Max Window Open 143KIAS - Service Ceiling 14700ft - MTOW 1670lb/757kg - MRW 1677.9lb/760.6kg | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC/GND 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE & FUEL - Model Avro Lycoming O-235-L2C - Max Power 110BHP@2550RPM - Max RPM 2550RPM - Displacement 233.3in³/3.82L - Block Fuel Burn 25L/hr - Hold Fuel Burn @ 55% 17L/hr - Taxi fuel allowance 5L - Min VFR Reserve 30min - Total Fuel 26.0USG/98.4L - Usable Fuel 24.5USG/93.0L - Fuel Type AVGAS100LL - Max Oil QTY 6QT/5.7L - Min Oil Pressure 25psi - Max Oil Pressure 100psi |
| --- | --- | --- |

| VH-NKJ |  |
| --- | --- |
| WEIGHT & BALANCE - MTOW 1670 lb - MTOW 757 kg - MRW 1677.9 lb - MRW 760.6 kg - BEW 1176.8 lb - BEW 533.8 kg - Max load@full fuel 344.45 lb - Max load@full fuel 156.24 kg - Max Area 1 120 lb - Max Area 1 54 kg - Max Area 2 39.7 lb - Max Area 2 18 kg - Max Area 1+2 120 lb - Max Area 1+2 54 kg - Arm 30.8 in - Arm 783 mm - Moment 36290 lb/in - Moment 418059 kg/mm Example @ MRW 760.6 kg. BEW; 533.8 kg. Row 1; 130 kg. Area 1; 20 kg. Area B; 9.5 kg. Usable Fuel; 66.96 kg 93.0 L 24.5 USG. TOTAL; 760.26 kg. | CRUISE PERFORMANCE 2400 RPM; 2300 RPM; 2200 RPM; 2100 RPM. Altitude; KTAS; L/hr; NM/L; KTAS; L/hr; NM/L; KTAS; L/hr; NM/L; KTAS; L/hr; NM/L. 2000; 101; 23.2; 4.35; 96; 20.5; 4.68; 91; 18.2; 5.00; 86; 16.3; 5.28. 4000; 101; 21.7; 4.65; 95; 19.4; 4.90; 90; 17.5; 5.14; 85; 16.0; 5.31. 6000; 100; 20.5; 4.88; 95; 18.6; 5.11; 89; 16.7; 5.33; 84; 15.2; 5.53. 8000; 99; 19.8; 5.00; 94; 17.9; 5.25; 89; 16.3; 5.46; 83; 14.8; 5.61. 10000; 98; 19.0; 5.16; 93; 17.1; 5.44; 88; 16.0; 5.50; 82; 14.8; 5.54. 12000; 97; 18.2; 5.33; 92; 16.7; 5.51; 87; 15.6; 5.58; 81; 14.4; 5.63. ISA, 1670lb, nil wind, speed fairings (else -2kt), mixture lean. Source: Cessna 152 Flight Manual 5-14. RANGE PROFILE 75% power; 65% power; 55% power; 45% power. Altitude; NM; KTAS; Min; NM; KTAS; Min; NM; KTAS; Min; NM; KTAS; Min. 0; 343; 100; 205; 372; 94; 237; 403; 87; 280; 430; 77; 336. 2000; 345; 204; 373; 236; 404; 278; 428; 333. 4000; 346; 103; 204; 374; 97; 235; 404; 89; 276; 427; 78; 329. 6000; 348; 203; 375; 233; 404; 274; 422; 324. 8000; 350; 107; 202; 376; 100; 232; 404; 91; 271; 419; 79; 319. 10000; 366; 218; 377; 228; 402; 266; 413; 313. ISA, 1670lb, nil wind, speed fairings (else -2kt), mixture lean. Allows for engine start, taxi, take-off, climb, 45 min reserve. Source: Cessna 152 Flight Manual 5-15, 5-17. |