| PRE-START - Park Brake ON - Doors LOCKED - Master Switch ON - Avionics OFF - Throttle ¼ inch OPEN if engine is COLD, prime - Fuel Pump ON - Mixture RICH TL FLOW IND… - Fuel Pump OFF - Mixture IDLE CUT OFF - “Clear Prop” ANNOUNCE - Starter ENGAGE - Mixture ADVANCE SMOOTHLY AFTER START - Oil Pressure RISING/GREEN - Throttle 1000 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp ≥100°F - Park Brake ON - Throttle 1800 RPM - Suction Gauge NORMAL - Ignition 1 CHECK ≤125 RPM - Ignition 2 CHECK ≤125 RPM - Ignition 1/2 Δ ≤50 RPM - Ammeter CHECK CHARGE - Idle CHECK ≥600 RPM - Throttle 1000 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture RICH - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Flaps SET 10° - Fuel BOTH&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK - T/O Safety/DEP/TEM BRIEF HOLDING READY - Lights AS REQ - Camera XPNDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture FULL RICH - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Flaps 0° - Trim SET TKOF - Lights OFF SHUT DOWN - Park Brake ON - Trim SET TKOF - Throttle MIN RPM - Avionics OFF - Mixture IDLE CUT OFF - Mags OFF - Master OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE & CLEAN |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 65KIAS - Troubleshoot F.A.S.T. - Fuel pump ON - Fuel selectors BOTH - Air CARB HEAT N/A - Switches MAGNETO BOTH - Switches MASTER ON - Throttle CYCLE - Turn over ENGINE START - Field SELECT - Wind - Obstacles - Size - Shape - Surface - Slope - Sun - Shoots - Surroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - Height RECOVER ≥3000 AGL - Aircraft CONFIG, TRIM - Security HARNESS, OBJECTS - Engine OPS NORMAL, P/T - Location UNPOPUL, FLD AVBL - Lookout TRAFFIC - Listen CHECK FREQ SPEEDS - Vne 163KIAS - Vno 129KIAS - Va 1111kg (MTOW) 99KIAS - Va 728kg 82KIAS - Vfe 0-10° 110KIAS - Vfe 10-30° 85KIAS - Vs0 33KIAS - Vs1 44KIAS - Vx 60KIAS - Vy 79KIAS - Best Glide 65KIAS - Max crosswind 15KT - Max tailwind 10KT | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - Southport CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE - Model Lycoming IO-360-L2A - Max RPM (cont.) 2400RPM - Block Fuel Burn 33L/hr - Min VFR Reserve 45min - Fuel AVGAS100LL - Min Oil Temp 100°F - Max Oil QTY 8 QTZ - Min Oil QTY 6 QTZ |
| --- | --- | --- |