| PRE-START - Pre-flight CMPL - Park Brake ON - Beacon light ON - Avionics OFF - Circuit Breakers CHECK - Autopilot OFF - Cowl Flaps OPEN - Fuel Selector !BOTH - Throttle Friction SET - If engine is COLD, prime - Fuel Primer STROKE x3 - Carburettor Heat OFF - Throttle ∙10mm OPEN - Np ∙FULL FINE - Fuel Mixture ∙FULL RICH - Propeller Area CHECK - “Clear Prop” ANNOUNCE - Master Switch ON - Starter ENGAGE AFTER START - Oil Pressure 30-100psi - Throttle ∙1000 RPM - Fuel Mixture ∙LEAN - Avionics ON - NAV Lights ON - Transponder SDBY - Flaps VERIFY/RETRACT - Cockpit config CFM | RUN-UP - Fuel Mixture ∙FULL RICH - Park Brake ON - Throttle ∙1700 RPM - Ignition R CHECK ≤150 RPM - Ignition L CHECK ≤150 RPM - Ignition R/L Δ ≤50 RPM - Carb Heat ON→OFF(RPM drop) - Vacuum Gauge CHECK - Np ∙CYCLE - Alt. Static CHECK - Idle CHECK ≥600 RPM - Autopilot CHECK PRE TAKE-OFF - T rim SET TKOF - M ixture ∙FULL RICH - P rop Pitch ∙FULL FINE - F light CTRL FREE,CORRECT - F laps SET TKOF 10° - F uel !BOTH,QTY,PRIMER LOCK - I nstruments CHECK - S witches AS REQ - H atches & H arnesses SECURE - T/O Safety/DEP/TEM !BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - B rakes PRESSURE&OFF - U ndercarriage !DOWN&LOCKED - M ixture ∙RICH - F uel !SELECT BOTH,SUFF QTY - I nstruments CHECK - S witches AS REQ - H atches & H arnesses CHECK AFTER LANDING - Carburettor Heat OFF - Flaps RETRACT 0° - Cowl Flaps OPEN - Transponder SDBY - Fuel Mixture ∙LEAN - Landing Light OFF SHUT DOWN - Park Brake ON - Trim SET TKOF - Throttle ∙MIN RPM - Avionics OFF - Fuel Mixture ∙IDLE CUTOFF - Magnetos OFF - Master Switch OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 70KIAS - Troubleshoot F.A.S.T. - F uel primer LOCKED - F uel selector BOTH - A ir CARB HEAT ON - S witches ON - T hrottle CYCLE - T urn over ENGINE START - Field SELECT - W ind - O bstacles - S ize - S hape - S urface - S lope - S un - S hoots - S urroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - H eight RECOVER ≥3000 AGL - A ircraft CONFIG, TRIM - S ecurity HARNESS, OBJECTS - E ngine OPS NORMAL, P/T - L ocation UNPOPUL, FLD AVBL - L ookout TRAFFIC - L isten CHECK FREQ SPEEDS & LIMITS - Vne 176KIAS - Vno 141KIAS - Va2950lb 110KIAS - Va2450lb 100KIAS - Va1950lb 89KIAS - Vfe0-10° 140KIAS - Vfe10-40° 95KIAS - Vs0 48KIAS - Vs1 53KIAS - Vx 59KIAS - Vy 80KIAS - Best Glide 70KIAS - Max crosswindlanding 20KT - Max crosswindtakeoff 15KT - Load Factorcruise +3.8G/-1.52G - Load Factorflaps +2.0G - Angle of Bank 60° - Service Ceiling 17700ft | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE & FUEL - Model Continental 6cyl - Max Power 230BHP@2600RPM - Max RPM 2600RPM - Block Fuel Burn 55L/hr - Min VFR Reserve 30min - Total Fuel 80.0USG/303.0L - Usable Fuel 75.0USG/285.0L - Fuel Type AVGAS100LL - Max Oil Temp 116°C/460°F - Min Oil QTY 9QT/8.5L - Max Oil QTY 12QT/11.3L - Min Oil Pressure 10psi - Max Oil Pressure 100psi |
| --- | --- | --- |

| VH-TIN |  |
| --- | --- |
| WEIGHT & BALANCE - MTOW 2950 lb - MTOW 1338.1 kg - MLW 2950 lb - MLW 1338.1 kg - Max load@full fuel 664.9 lb - Max load@full fuel 301.72 kg - Max Area A 120 lb - Max Area A 54.55 kg - Max Area B+hatshelf 80 lb - Max Area B+hatshelf 36.36 kg - Max hatshelf 25 lb - Max hatshelf 11.36 kg - BEW 1835.1 lb - BEW 832.38 kg - Arm 36.5 in - Arm 926 mm - Moment 66908 lb/in - Moment 770892 kg/mm MTOW 2950lb example. Row 1; 160 kg. Row 2; 160 kg. Area A; (max) 54.55 kg. Area B; (max) 36.36 kg. Fuel; 94.81 kg 131.68 L 34.86 USG. | CRUISE PERFORMANCE 75% power; 65% power; 55% power. Altitude; KTAS; NM/L; KTAS; NM/L; KTAS; NM/L. 3000; 140; 2.63; 132; 2.89; 122; 3.11. 6500; 144; 2.74; 136; 3.00; 125; 3.18. 10000; 140; 3.08; 128; 3.26. Standard conditions, nil wind. ISA-20 temp (-13°C); ISA temp (7°C); ISA+20 temp (27°C). RPM; MP; %BHP; KTAS; L/hr; %BHP; KTAS; L/hr; %BHP; KTAS; L/hr. 2450; 23; 77; 142; 54.0; 74; 143; 52.1. 22; 75; 138; 53.2; 73; 139; 50.9; 70; 140; 49.4. 21; 71; 135; 49.8; 69; 136; 48.3; 66; 137; 46.7. 20; 67; 132; 47.1; 65; 133; 45.2; 62; 133; 44.1. 2400; 23; 78; 139; 55.1; 75; 141; 52.8; 72; 142; 50.9. 22; 74; 137; 51.7; 71; 138; 49.8; 69; 139; 48.3. 21; 70; 134; 49.0; 67; 135; 47.1; 65; 136; 45.6. 20; 66; 131; 46.0; 63; 132; 44.5; 61; 132; 43.3. 2300; 23; 74; 137; 51.7; 71; 138; 49.8; 69; 139; 48.3. 22; 70; 134; 49.1; 67; 135; 47.5; 65; 136; 46.0. 21; 66; 131; 46.4; 64; 132; 44.8; 62; 133; 43.3. 20; 62; 128; 43.7; 60; 128; 42.2; 58; 128; 41.0. Pressure Altitude 4000ft @ 2950lb. |