| PRE-START - Park Brake ON - Circuit Breakers CHECK - Master Switch KEY ENGINE - Choke if cold engine ON - Fuel Pump ON→PRESSURE - Fuel Pump OFF - Throttle MIN + ½ TURN - Ignition 1 & 2 ON - Propeller Area CHECK - “Clear Prop” ANNOUNCE - Starter ENGAGE AFTER START - Oil Pressure RISING/GREEN - Choke OFF - Throttle 2000 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp ≥50°C - Park Brake ON - Choke OFF - Throttle 3600 RPM - Ignition 1 CHECK ≤125 RPM - Ignition 2 CHECK ≤125 RPM - Ignition 1/2 Δ ≤50 RPM - Carburettor Heat ON→OFF - Idle CHECK ≥1250 RPM - Throttle 2000 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture CHOKE OFF - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Flaps SET TKOF - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK - T/O Safety/DEP/TEM BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture CHOKE OFF - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Lights OFF - Flaps & Trim SET TKOF SHUT DOWN - Park Brake ON - Flaps & Trim SET TKOF - Throttle MIN RPM - Avionics OFF - Ignition 1 OFF ...pause 2 seconds - Ignition 2 OFF - Master Switch KEY OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 65KIAS - Troubleshoot F.A.S.T. - Fuel pump ON - Fuel selectors 3xON - Air CARB HEAT ON - Switches IGNITION 2xON - Switches MASTER ON - Throttle CYCLE - Turn over ENGINE START - Field SELECT - Wind - Obstacles - Size - Shape - Surface - Slope - Sun - Shoots - Surroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - Height RECOVER ≥2000 AGL - Aircraft CONFIG, TRIM - Security HARNESS, OBJECTS - Engine OPS NORMAL, P/T - Location UNPOPUL, FLD AVBL - Lookout TRAFFIC - Listen CHECK FREQ SPEEDS - Vne 124KIAS - Va 95KIAS - Vfe 81KIAS - Vs0 36KIAS - Vs1 43KIAS - Vx 60KIAS - Vy 65KIAS - Best Glide 65KIAS - Max wind (RWY HDG) 25KT - Max crosswind 15KT - Max tailwind 6KT | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - Southport CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE - Model Rotax 912ULS - Max RPM (5 mins) 5800RPM - Max RPM (cont.) 5500RPM - Block Fuel Burn 20L/hr - Min VFR Reserve 45min - Fuel 95RON¦AVGAS100LL - Min Oil Temp 50°C - Max Oil QTY 2.6L - Min Oil QTY 2.1L |
| --- | --- | --- |

| 24-4844 - Total Fuel Capacity 85.1L - Total Fuel Capacity 61.3kg - Total Usable Fuel 83L - Total Usable Fuel 59.8kg - Basic Empty Weight 288.5kg - Empty Arm 244.8mm - Empty Arm 18.83% - Permitted CG min. 259.08mm - Permitted CG min. 20% - Permitted CG max. 363.22mm - Permitted CG max. 28% - Max Take-Off Weight 560kg - Min Crew Weight 55kg - Max Baggage Weight 20kg - Max LF (no flaps) +4G - Min LF (no flaps) -2G - Max LF (flaps) +2G - Min LF (flaps) 0G - Max Angle of Bank 60° - Max endurance 6 hours - Max range 534NM - Service ceiling 14760ft - Propeller FITI ECO ⌀1580mm | 24-5350 - Total Fuel Capacity 85.0L - Total Fuel Capacity 61.2kg - Total Usable Fuel 83L - Total Usable Fuel 59.8kg - Basic Empty Weight 303.5kg - Empty Arm 266.0mm - Empty Arm 20.46% - Permitted CG min. 259.08mm - Permitted CG min. 20% - Permitted CG max. 363.22mm - Permitted CG max. 28% - Max Take-Off Weight 560kg - Min Crew Weight 54.9kg - Max Baggage Weight 20kg - Max LF (no flaps) +4G - Min LF (no flaps) -2G - Max LF (flaps) +2G - Min LF (flaps) 0G - Max Angle of Bank 60° - Max endurance 6 hours - Max range 534NM - Service ceiling 14760ft - Propeller FITI ECO ⌀1580mm | 24-8881 - Total Fuel Capacity 86.0L - Total Fuel Capacity 61.9kg - Total Usable Fuel 85L - Total Usable Fuel 61.2kg - Basic Empty Weight 303.8kg - Empty Arm 304.0mm - Empty Arm 24.18% - Permitted CG min. 260mm - Permitted CG min. 20% - Permitted CG max. 442mm - Permitted CG max. 34% - Max Take-Off Weight 560kg - Min Crew Weight 54kg - Max Baggage Weight 20kg - Max LF (no flaps) +4G - Min LF (no flaps) -2G - Max LF (flaps) +2G - Min LF (flaps) 0G - Max Angle of Bank 60° - Max endurance 6 hours - Max range 534NM - Service ceiling 14760ft - Propeller FITI ECO ⌀1580mm |
| --- | --- | --- |