| PRE-START - Park Brake ON - Battery Isolator OFF - Master Switch ON - Throttle IDLE - Fuel Pumps A+B ON - Lane A+B ON - Start Switch HOLD UP - Lane A/B Lights Check OFF - Propeller Area CHECK - “Clear Prop” ANNOUNCE - Start Button PRESS AFTER START - Oil Pressure RISING/GREEN - Throttle 2000 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp ≥50°C - Park Brake ON - Throttle 3000 RPM - Battery Charge CHECK ON - Idle CHECK ≥1400 RPM - Throttle 2000 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture CHOKE OFF - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Flaps SET TKOF - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK - Safety/DEP BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture N/A - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Lights OFF - Flaps & Trim SET TKOF SHUT DOWN - Park Brake ON - Flaps & Trim SET TKOF - Throttle IDLE - Avionics OFF - Fuel Pumps A/B OFF - Lane A OFF - Lane B OFF - Master Switch KEY OFF - Battery Isolator ON AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 54 KIAS - Troubleshoot F.A.S.T. - Fuel Pumps ON - Fuel selectors ON - Air Alternate - Switches LANEs A/B ON - Switches MASTER ON - Throttle CYCLE - Turn over ENGINE START - Field SELECT - Wind - Obstacles - Size - Shape - Surface - Slope - Sun - Shoots - Surroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - Height RECOVER ≥2000 AGL - Aircraft CONFIG, TRIM - Security HARNESS, OBJECTS - Engine OPS NORMAL, P/T - Location UNPOPUL, FLD AVBL - Lookout TRAFFIC - Listen CHECK FREQ SPEEDS - Vne 124KIAS - Va 101KIAS - Vfe 80KIAS - Vs0 32KIAS - Vs1 42KIAS - Vx 46KIAS - Vy 52KIAS - Best Glide 54KIAS - Max crosswind 14KTS | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - Southport CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE - Model Rotax 912ULS - Max RPM (5 mins) 5800RPM - Max RPM (cont.) 5500RPM - Block Fuel Burn 20L/hr - Min VFR Reserve 45min - Fuel 95RON¦AVGAS100LL - Min Oil Temp 50°C - Max Oil QTY 2.6L - Min Oil QTY 2.1L |
| --- | --- | --- |