| PRE-START - Park Brake ON - Circuit Breakers CHECK - Master/Mags ON - Avionics OFF - Fuel Valve ON/FLUSH - Throttle ¼ inch OPEN - Pitch FULL FINE if engine is COLD, prime - Mixture FULL RICH - Electric Pump ON 3 SECS - Mixture IDLE CUT OFF - Electric Pump OFF - Ignition 1/2 ON - “Clear Prop” ANNOUNCE - Starter ENGAGE - Mixture ADVANCE SMOOTHLY - Mixture LEAN FOR TAXI AFTER START - Oil Pressure RISING/GREEN - Throttle 1000 RPM - Avionics ON SYMBOLS Δ Difference ¦ OR & AND ≤ Less than or equal to ≥ Greater than or equal to | RUN-UP - Oil Temp ≥120°F - Park Brake ON - Throttle 1800 RPM - Ignition 1 CHECK ≤200 RPM - Ignition 2 CHECK ≤200 RPM - Ignition 1/2 Δ ≤50 RPM - Prop Cycle x2 ≤300 RPM - Alternate Air ON→OFF - Idle CHECK ≥500 RPM - Throttle 1000 RPM PRE TAKE-OFF - Trim SET TKOF - Mixture FULL RICH - Prop Pitch FULL FINE - Flight CTRL FREE&CORRECT - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK - T/O Safety/DEP/TEM BRIEF HOLDING READY - Lights STROBE ON - Camera XPDR 3000 ALT - Action READY, REQ CLR | PRE LANDING - Brakes PRESSURE&OFF - Undercarriage DOWN&LOCKED - Mixture FULL RICH - Fuel ON&SUFF QTY - Instruments CHECK - Switches AS REQ - Hatches & Harnesses CHECK AFTER LANDING - Lights OFF - Flaps & Trim SET TKOF SHUT DOWN - Park Brake ON - Trim SET TKOF - Throttle MIN RPM - Avionics OFF - Mixture IDLE CUT OFF - Mags OFF - Master OFF AFTER SHUT DOWN - MR if last FLT of day CLOSE - Hobbs/Tach REC BOOK - Hobbs/Tach REC OFFICE - Aircraft SECURE |
| --- | --- | --- |

| EMERG FORCED LDG - Aviate, Navigate, Comm. - Best Glide 65KIAS - Troubleshoot F.A.S.T. - Fuel pump ON - Fuel Valve ON - Air ALTERNATE AIR ON - Switches MAGS 2xON - Switches MASTER ON - Throttle CYCLE - Turn over ENGINE START - Field SELECT - Wind - Obstacles - Size - Shape - Surface - Slope - Sun - Shoots - Surroundings/Buildings - Communicate MAYDAY - Passenger BRIEF - Aircraft SECURE | STALL ENTRY - Clearing Turn AS REQ - Height RECOVER ≥3000 AGL - Aircraft CONFIG, TRIM - Security HARNESS, OBJECTS - Engine OPS NORMAL, P/T - Location UNPOPUL, FLD AVBL - Lookout TRAFFIC - Listen CHECK FREQ SPEEDS - Vne 174KIAS - Va (aero) 115KIAS - Va (normal) 93KIAS - Vno 139KIAS - Vs 47KIAS - Vx 51KIAS - Vy 72KIAS - Best Glide 65KIAS - Max crosswind 17KT | FREQUENCIES - YBAF ATIS 120.90MHz - YBAF SMC 119.90MHz - YBAF TWR (NORTH) 123.60MHz - YBAF TWR (SOUTH) 118.10MHz - YBAF CTAF 118.10MHz - YBAF AWIS 128.40MHz - YBAF PAL 118.10MHz - BN CEN Train. 125.70MHz - BN CEN 45NM N. 129.00MHz - BN CEN 14NM S. 119.50MHz - YHEC CTAF 119.00MHz - Southport CTAF 119.00MHz - YAMB ATIS 123.30MHz - GEN Air Distress 121.50MHz ENGINE - Model Lycoming AEIO-360 - Max RPM 2700RPM - Block Fuel Burn 40L/hr - Min VFR Reserve 45min - Fuel AVGAS100LL - Min Oil Temp 120°F - Max Oil QTY 8 QTZ - Min Oil QTY 4 QTZ |
| --- | --- | --- |

VH-NQX - Total Fuel Capacity 152L - Total Fuel Capacity 110kg - Total Usable Fuel 152L - Total Usable Fuel 110kg - Basic Empty Weight 616kg - Empty Arm 13.3in - COG Norm. 884kg 14.7-18.5in - COG Norm. ≤703kg 11.5-18.5in - COG Aero. 816kg 13.5-18.5in - COG Aero. ≤703kg 11.5-18.5in - MTOW Normal Cat. 884kg - MTOW Aerobatic Cat. 816kg - Max Baggage Norm. 45kg - Max LF Norm. 884Kg +3.8G - Min LF Norm. 884Kg -1.52G - Max LF Aero. 816kg +6G - Min LF Aero. 816kg -5G - Max endurance 3.8 hours - Max range 418NM - Service ceiling 16000ft - Propeller Hartzell HC-C2YR ⌀1930mm - Max Inverted Time* 2 min - Tail Slides/Lomcevaks N/A *One minute upright time is required to fill header tank