| COLD START - Fuel Mixture ∙FULL RICH - Master Switch ON - Alternator ON - Fuel Pump ON - Throttle ∙⅛ OPEN - Fuel Pump OFF - Propeller Pitch ∙FULL FINE - Flaps RETRACT 0° - “Clear Prop” ANNOUNCE - Starter ENGAGE WARM START - Fuel Mixture ∙LEAN - Throttle ∙⅛ OPEN - Propeller Pitch ∙FULL FINE - Master Switch ON - Alternator ON - Flaps RETRACT 0° - “Clear Prop” ANNOUNCE - Starter ENGAGE - Fuel Mixture ∙FULL RICH AFTER START - Oil Pressure IDLE @ 25psi - Fuel Mixture ∙LEAN - Avionics ON - NAV Lights ON - Transponder SDBY - Flaps VERIFY/RETRACT - Cockpit config CFM - RUN-UP - Fuel Mixture ∙FULL RICH - Park Brake ON - Throttle ∙1700 RPM - Ignition R CHECK ≤50 RPM - Ignition L CHECK ≤50 RPM - Np ∙CYCLE - Idle CHECK ≥600 RPM - Autopilot CHECK - T.M.P.F.I.S.H. | - SPEEDS & LIMITS - Vne 200KIAS - Vno 156KIAS - Va 130KIAS - Vfe 100KIAS - Vs0 51KIAS - Vs1 62KIAS - Vx 70KIAS - Vy 95KIAS - Best Glide 78KIAS - Service Ceiling 19500ft - AEROBATICS - Loop, Horiz. Eight 122-165KIAS - Immelman Turn 130-165KIAS - Aileron/Barrel Roll 105-165KIAS - Snap Roll 70-95KIAS - Vertical Roll 156-165KIAS - Split-S 87-96KIAS - ENGINE & FUEL - Model Superior IO-400 - Block Fuel Burn 40L/hr - Min VFR Reserve 30min - Total Fuel 188.0L - Fuel Type AVGAS100LL - Max Oil Temp 118°C/245°F - Min Oil QTY 2QT/1.9L - Max Oil QTY 8QT/7.6L - Min oper. Oil Pressure 55psi - Max oper. Oil Pressure 95psi - WEIGHT & BALANCE - MTOW 930 kg - BEW 579 kg - Arm 2068.0 mm - Moment 1196757 kg/mm - Full Fuel (188L) 135.36 kg - Max load@full fuel 215.64 kg - Max Baggage 45.4 kg - Crew Arm 2536.0 mm - Fuel Arm 2082.0 mm - Baggage Arm 3288.8 mm - CG Limit 2084.8-2241.3 mm |
| --- | --- |